Model JK-1 and Model 111F

Instruction Manual

Vulcan Kilns, LLC

7623 Clyo Road

Centerville, Ohio 45459, U.S.A.

Phone (937) 433-1833

Email: vulcankilns@sbcglobal.net

Kiln Specifications

Model Max. temperature Volts Amp Watts NEMA Breaker size

JK-1 2250°F 115 10 1150 5-15 15

111F 2250°F 115 14 1610 5-15 15

Kiln Description

Thank you for purchasing a Vulcan Kiln. Vulcan Kilns have been produced since 1958.

We use the highest quality materials to provide years of reliable service. The JK-1 and

111F kilns are completely manual kilns. You are responsible to monitor and control kiln

temperature. Please determine your firing plan before starting. JK-1 and 111F kilns are

primarily used for firing ceramic (clay), glass and P.M.C. (metal Clay), up to 2250°F.

They may also be used for other applications, such as heat treat and testing.

Kiln Set-up and Safety

Please follow these steps for a safe kiln firing. Kiln exterior temperature could reach

350°F during operation, so use care when operating. Set kiln on heat resistant blocks or

bricks during use. Keep kiln at least 12” away from any flammables. Keep cord away

from side of kiln. Use kiln in a well-ventilated and dry area and keep children away.

Closely monitor kiln temperature to avoid exceeding either kiln maximum temperature

(2250°F) or desired firing temperature. Exceeding either may cause damage to the fired

item or kiln. Do not leave kiln unattended during firing. Allow to cool before opening.

Contact with heating elements is to be avoided, as elements carry live current when in

operation. Always unplug kiln when not in use.

Calibrating the thermocouple

The analog pyrometer on your kiln can be easily calibrated. Using a small flathead

screwdriver, slowly turn the white screw head on the pyrometer faceplate until the

indicating needle is at the correct temperature. NOTE: a 75°F line is marked on the meter

for use in calibrating at a normal room temperature, use this as your guide.

Firing Directions

For best results, it is recommended to install a kiln shelf with 3 ½” posts underneath.

This allows for a more successful firing as the kiln floor may be slightly cooler. Coat top

of kiln shelf with appropriate surface treatment i.e. kiln wash or shelf paper as needed for

the particular item being fired. The JK-1 and 111F are used to fire a number of different

materials. Please research each material to be fired and determine all firing times and

temperatures before firing.

Contact your material supplier if you are unsure as to what temperature or cone # to fire

kiln. Test fire your kiln empty to desired temperature in order to determine firing times.

The JK-1 and 111F are fast firing kilns, so acclimating yourself to the firing speeds will

be very beneficial. This will help you to avoid over-fire. Your kiln has an analog

pyrometer to indicate temperature and an infinite switch to control it. Once the kiln has

been loaded, install lid and remove peephole plug for ventilation. Turn kiln on low for

desired amount of time. As a general rule, the thicker the clay, the slower you would

need to fire it. Handmade sculpture or thick pottery would need to be on low setting at

least 2 hours. Next, turn kiln on medium setting (3-4) for approximately ½ hour.

Heating items too quickly may cause cracking of your items. Install peephole plug when

kiln reaches around 1100°F. Closely watch temperature and turn kiln to high

temperature, if needed, until maximum firing temperature for your material is achieved.

Turn switch to off position, unplug and allow kiln to cool before opening.

Kiln Characteristics

During initial firing some smoke may be seen emanating from the elements and infinite

switch. This is normal. New elements have some oils from manufacture which will burn

off quickly. Infinite switches pulse on and off. During this cycle, a “pop”, or visual

spark, may be seen from the switch, due to switch contacts closing and opening, this is

also a normal characteristic.  Kiln brick is light and fragile.  It is a very insulating brick.  Due to

process of manufacture, you may see air voids, resembling bubbles, which gives the brick its

insulating qualities and lightness.  The brick also may have surface cracks or spidering, this is

also normal.  Even though the brick is very insulating, the surface of the kiln will be hot during

use, so do not touch.  During firing, cracks in the brick may occur during use and is a result of

expansion and contraction, this too is normal.

Warranty

The Vulcan JK-1 and 111F kilns have a 2 year limited warranty. This covers any defects

in material or workmanship. This does not cover thermocouple or damage to the kiln

from misuse or overfire. Please contact Vulcan Kilns or your distributor for warranty

claims.

Witness Cones

If your firing is to a cone number (#), use the following cone chart for guidance in

determining what temperature to operate your kiln. Remember a cone # is a combination

of time and temperature and not just temperature achieved. Using a large witness cone is

a good way to assure your load has received the proper heatwork.

Temperature Equivalents (°F)

Orton Pyrometric Cones

Heating rate °F per hour (last 180°F of firing)

Self Supporting Cones

| Cone | 108 | 270 |
| --- | --- | --- |
| 022 | 1087 | 1094 |
| 021 | 1112 | 1143 |
| 020 | 1159 | 1180 |
| 019 | 1252 | 1283 |
| 018 | 1319 | 1353 |
| 017 | 1360 | 1405 |
| 016 | 1422 | 1465 |
| 015 | 1456 | 1504 |
| 014 | 1485 | 1540 |
| 013 | 1539 | 1582 |
| 012 | 1582 | 1620 |
| 011 | 1607 | 1641 |
| 010 | 1657 | 1679 |
| 09 | 1688 | 1706 |
| 08 | 1728 | 1753 |
| 07 | 1789 | 1809 |
| 06 | 1828 | 1855 |
| 05 1/2 | 1859 | 1877 |
| 05 | 1888 | 1911 |
| 04 | 1945 | 1971 |
| 03 | 1987 | 2019 |
| 02 | 2016 | 2052 |
| 01 | 2046 | 2080 |
| 1 | 2079 | 2109 |
| 2 | 2088 | 2127 |
| 3 | 2106 | 2138 |
| 4 | 2124 | 2161 |
| 5 | 2167 | 2205 |
| 5 1/2 | 2197 | 2237 |
| 6 | 2232 | 2269 |

CAUTION

• Surface of kiln is HOT during use

• Support kiln with non-flammable

materials (i.e. bricks or blocks)

• Keep at least 12” clearance around kiln

• Keep children away from kiln

• Use kiln in a well-ventilated and dry area

• Keep switch in “OFF” position when not in

use and unplug kiln

• Never fire kiln unattended

• Closely monitor kiln temperature to avoid

overfire

***Please Post Near Kiln***